RELIABILITY TEST REPORT

TEST ITEM : 1. ELECTRICAL

2. MECHANICAL

3. ENVIRONMENTAL

SERIES  NO. : CF32 SERIES

TEST  EQUIPMENT : 1. INSERTION & REMOVAL APPARATUS

2. ELECTRONIC MEASURING APPARATUS

3. ENVIRONMENTAL APPARATUS

DATE  OF  TESTING : 11/25/2011”

TEST  DEPART :         R&D              TESTER : Claire

CONTENT : ATTACHED

REVIEWED :     Jerry      APPROVED :   Francis      VERIFIED :   Clarie   .

- ELECTRICAL PERFORMANCE :

|  | ITEM | TEST CONDITION | REQUIREMENT | TEST RESULT |  |
| --- | --- | --- | --- | --- | --- |
| 1-1 | # Dielectric strength | Test between adjacent contacts with a voltage of 500 VAC for 1 minute at Sea level. Test as per EIA364-20 Method B | # No Damage | Sample | 500 V 1 minute |
|  |  |  |  | 1 | OK |
|  |  |  |  | 2 | OK |
|  |  |  |  | 3 | OK |
|  |  |  |  | 4 | OK |
|  |  |  |  | 5 | OK |
| 1-2 | # Insulation resistance | After 500 VDC for 1 minute , measure the insulation resistance between the adjacent contacts. Test as per EIA364-21 | # More than 500 M | Sample | 500 M min |
|  |  |  |  | 1 | ＞500 M |
|  |  |  |  | 2 | ＞500 M |
|  |  |  |  | 3 | ＞500 M |
|  |  |  |  | 4 | ＞500 M |
|  |  |  |  | 5 | ＞500 M |
| 1-3 | # Contact Resistance | Measured at 20 mV maximum open circuit at 100mA .Mated test contacts must be in a connector housing. Test as per EIA364-23 | Less than 20 mΩ | Sample | 20 m. max. |
|  |  |  |  | 1 | 9.8 m |
|  |  |  |  | 2 | 10.7 m |
|  |  |  |  | 3 | 9.2 m |
|  |  |  |  | 4 | 10.2m |
|  |  |  |  | 5 | 10.6m |

2. MECHANICAL PERFORMANCE :

|  | ITEM | TEST CONDITION | REQUIREMENT | TEST RESULT |  |
| --- | --- | --- | --- | --- | --- |
| 2-1 | Contact retaining force in insulator | The end of terminal shall be pulled in a perpendicular to base housing at a maximum rate of 25 mm per minute. Test as per EIA 364-29 | More than 0.3Kgf | Sample | 0.3 Kgf min. |
|  |  |  |  | 1 | 0.386 Kgf |
|  |  |  |  | 2 | 0.438 Kgf |
|  |  |  |  | 3 | 0.426 Kgf |
|  |  |  |  | 4 | 0.431 Kgf |
|  |  |  |  | 5 | 0.397 Kgf |
| 2-2 | FFC/FPC Retention Force | Apply axial load to FFC/FPC by operating at the speed rate of 25 mm per minute. | 0.03 Kgf /Pin min. | Sample | 0.03Kgf/Pin min. |
|  |  |  |  | 1 | 0.060 Kgf |
|  |  |  |  | 2 | 0.060 Kgf |
|  |  |  |  | 3 | 0.061 Kgf |
|  |  |  |  | 4 | 0.059 Kgf |
|  |  |  |  | 5 | 0.059 Kgf |

|  | ITEM | TEST CONDITION | REQUIREMENT | TEST RESULT |  |
| --- | --- | --- | --- | --- | --- |
| 2-3 | Fitting Nail Retention Force | Apply axial pull out of force at the speed of 25 mm per minute on the fitting nail assembled in the housing. | More than 0.3 Kgf | Sample | 0.3 Kgf min. |
|  |  |  |  | 1 | 0.434 Kgf |
|  |  |  |  | 2 | 0.398 Kgf |
|  |  |  |  | 3 | 0.471 Kgf |
|  |  |  |  | 4 | 0.456 Kgf |
|  |  |  |  | 5 | 0.437 Kgf |
| 2-4 | Durability | Mate applicable FFC/FPC and insert and withdraw actuator at the speed rate of 25 mm per minute. Times :Up to 30 cycles. | Appearance: No damage | Sample |  |
|  |  |  |  | 1 | OK |
|  |  |  |  | 2 | OK |
|  |  |  |  | 3 | OK |
|  |  |  |  | 4 | OK |
|  |  |  |  | 5 | OK |
|  |  |  | Contact Resistance: Less than 40 mΩ | Sample | 40 mΩ max. |
|  |  |  |  | 1 | 10.8 m |
|  |  |  |  | 2 | 10.7 m |
|  |  |  |  | 3 | 11.3 m |
|  |  |  |  | 4 | 10.2 m |
|  |  |  |  | 5 | 10.6 m |
|  |  |  | FFC/FPC Retention Force | Sample | 0.03Kgf/Pin min. |
|  |  |  |  | 1 | 0.055 Kgf |
|  |  |  |  | 2 | 0.057 Kgf |
|  |  |  |  | 3 | 0.051 Kgf |
|  |  |  |  | 4 | 0.053 Kgf |
|  |  |  |  | 5 | 0.062 Kgf |

3.ENVIRONMENTAL PERFORMANCE:

|  | ITEM | TEST CONDITION | REQUIREMENT | TEST RESULT |  |
| --- | --- | --- | --- | --- | --- |
| 3-1 | Temperature rise | The object of this test procedure is to detail a standard method to assess the current carrying capacity of mated battery connector contact. Test as per EIA364-70 Method B | 30°C max. | Sample | 30 ℃ max. |
|  |  |  |  | 1 | 27 ℃ |
|  |  |  |  | 2 | 28 ℃ |
|  |  |  |  | 3 | 26 ℃ |
|  |  |  |  | 4 | 27 ℃ |
|  |  |  |  | 5 | 27 ℃ |

|  | ITEM | TEST CONDITION | REQUIREMENT | TEST RESULT |  |
| --- | --- | --- | --- | --- | --- |
| 3-2 | Heat aging | Subject unmated connectors to temperature life at 85℃±2℃ for 96 hours. Test as per EIA 364 – 17 Test Condition Ⅲ Method A. | Appearance: No damage | Sample |  |
|  |  |  |  | 1 | OK |
|  |  |  |  | 2 | OK |
|  |  |  |  | 3 | OK |
|  |  |  |  | 4 | OK |
|  |  |  |  | 5 | OK |
|  |  |  | Contact resistance: Less than 40 mΩ | Sample | 40 mΩ max. |
|  |  |  |  | 1 | 11.7 m |
|  |  |  |  | 2 | 12.5 m |
|  |  |  |  | 3 | 11.6 m |
|  |  |  |  | 4 | 10.8 m |
|  |  |  |  | 5 | 10.3 m |
| 3-3 | Humidity | Subject unmated connectors to 96 hours at 40℃ with 90％ to 95％ RH. Test as per EIA 364 – 31 Method Ⅱ Test Condition A. | Appearance: No damage | Sample |  |
|  |  |  |  | 1 | OK |
|  |  |  |  | 2 | OK |
|  |  |  |  | 3 | OK |
|  |  |  |  | 4 | OK |
|  |  |  |  | 5 | OK |
|  |  |  | Contact resistance : Less than 40 mΩ | Sample | 40 mΩ max. |
|  |  |  |  | 1 | 10.5 m |
|  |  |  |  | 2 | 11.6 m |
|  |  |  |  | 3 | 10.2 m |
|  |  |  |  | 4 | 10.7 m |
|  |  |  |  | 5 | 9.5 m |
|  |  |  | Insulation resistance More than 500 M | Sample | 500 M min. |
|  |  |  |  | 1 | ＞500 M |
|  |  |  |  | 2 | ＞500 M |
|  |  |  |  | 3 | ＞500 M |
|  |  |  |  | 4 | ＞500 M |
|  |  |  |  | 5 | ＞500 M |

|  | ITEM | TEST CONDITION | REQUIREMENT | TEST RESULT |  |
| --- | --- | --- | --- | --- | --- |
| 3-4 | Temperature cycling | Subject unmated connectors shall be tested in accordance with EIA364–32 Test Condition Ⅰ (1)-55℃,30 minute (2)+25℃,5 minute (3)+85℃,30 minute (4)+25℃,5 minute consecutive 10 cycles.. | Appearance : No damage | Sample |  |
|  |  |  |  | 1 | OK |
|  |  |  |  | 2 | OK |
|  |  |  |  | 3 | OK |
|  |  |  |  | 4 | OK |
|  |  |  |  | 5 | OK |
|  |  |  | Contact resistance: Less than 40 mΩ | Sample | 40 mΩ max. |
|  |  |  |  | 1 | 14.7 m |
|  |  |  |  | 2 | 16.2 m |
|  |  |  |  | 3 | 15.8 m |
|  |  |  |  | 4 | 16.1 m |
|  |  |  |  | 5 | 13.6 m |
| 3-5 | Solder ability | Steam age 1 hour at 90℃～96℃ Solder time to be 5±1 seconds at 245℃, using unactivated flux. Test as per EIA364-52 | Minimum: 95% of immersed area | Sample |  |
|  |  |  |  | 1 | OK |
|  |  |  |  | 2 | OK |
|  |  |  |  | 3 | OK |
|  |  |  |  | 4 | OK |
|  |  |  |  | 5 | OK |
| 3-6 | Resistance to soldering heat | Soldering time: 10 second , 2times Soldering pot: 250~260°C max. | Appearance : No damage | Sample |  |
|  |  |  |  | 1 | OK |
|  |  |  |  | 2 | OK |
|  |  |  |  | 3 | OK |
|  |  |  |  | 4 | OK |
|  |  |  |  | 5 | OK |